Project Title – Design of IOT Architecture for Battery Charging Management System

The Problem Statement and Research gap identified

- In the context of current World Transportation system, the Electrical vehicles is rapidly increasing, so there is need to increase and stabilize the charging infrastructure.

- Now a Days EV Charging Infra is connected to Grids which leads to sudden disturbance of connected loads

- There is Lack real time or pre-estimated data of Power demand for generation stations

- High Installation and Operational Costs

- Inefficient Load Management and Grid Capacity Challenges.

- Remote data monitoring and User Interface.

Project Novelty

- Innovative IOT 5 layered Architecture.

- Integration of multiple energy sources.

- Central monitoring and controlling.

- Real-Time data processing and user interface.

- Cloud connectivity.

- Cost efficiency model.

- Scalability and Adaptability.

- Portable charging station.

Methodology and Proposed Systems

- Renewable and Nonrenewable energy sources Integration [Solar, ESS and Grid]

Solar – 430V – 1032 KW/day - 56.8% / day

ESS - 430V -   576 KW/day – 31.7% / day

Grid – 430V – 209 KW/day	- 11.5% / day

- PPC – Point of common coupling for the integration and automation

- 4 – Automation conditions.

- Solar – ON 	  ESS Charge 	 Grid OFF	 Load ON

- Solar – ON 	  ESS >80%   	 Grid ON 	 Load ON

- Solar – OFF 	  ESS ON       	 Grid OFF       Load ON

- Solar – OFF 	  ESS <20%   	 Grid ON 	  Load ON

- Solar – OFF 	 ESS Charging 	 Grid ON	 Load OFF

- Loads -

- 30 - Battery Swapping Station

12V – 0.12 kw / Charge

30 Batteries 3 times per day – 90 batteries charging per day – Total 54 kw / day

- 4 – DC Fast Charging Station.

[ Note – Every vehicle having different parameter and with minimum SOC]

Example –

| Cars | Voltage (V) | Power (kW) | Battery Capacity (kWh) | SOC Avaliable | Required Current | Charging Time in hr | Unit charge | Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 - Tata Nexon - EV | 400 | 30.2 | 30.2 | 20 | 75.5 | 2.47 | 10 | 745.83 |
| 2- MG ZS EV | 400 | 50 | 44.5 | 30 | 125 | 1.49 | 10 | 743.75 |
| 3-Tata Trigo EV | 400 | 16.8 | 26 | 50 | 42 | 4.40 | 10 | 739.58 |
| 4- Mahindra eVerito | 400 | 70 | 60 | 10 | 175 | 1.07 | 10 | 747.92 |

- Designed 5 layered IOT Architecture system

Device Layer: Integrate sensors and actuators at charging stations.

Network Layer: Select communication protocols (e.g., MQTT, HTTP).

Edge Computing Layer: Implement local processing for real-time data handling.

Cloud Layer: Utilize AWS for data storage and analytics.

Application Layer: Create user interfaces for monitoring and management.

- Algorithms

Solar MPPT

Battery Swapping Station – Charging

DC Fast Charging control calculation

- Real time monitoring with Sensors

- Node-red for intranet central monitoring data visualization [Dashboard HTTPS]

- AWS Connectivity for data storage

MQTT Test client

S3 Bucket for Data storage

SNS service for Email Alerts

Quickslights for Data visualization

Energy saving and Cost efficiency

Technical calculations –

Solar – 1032, ESS – 576, Grid – 209  Total – 1817 kw /day * 12rs unit – 21,804/day

Solar – 1032, ESS – 576 – Renewable energy  1608kw/day * 12rs unit – 19,296/day – Saving

Grid – 209 kw/day * 12rs / unit – 2,508/day Spending

Energy Saving –

Energy Saving Efficiency = (Renewable energy / Total Energy) X 100

(1608 / 1817) X 100

Energy Saving Efficiency = 88.6%

Cost saving Efficiency

Total cost - Grid cost = saving cost

21,804 – 2,508 = 19,296

Cost saving Efficiency = (Renewable Energy cost / Total cost) X 100

(19,295 / 21,804) X 100

Cost saving Efficiency = 88.5%

Revenue generation

- Battery Swapping Station – 30 no X 3 Times = 90 Batteries * 30rs charge = 2,700 rs

- DC Fast Charging Station – 4 no X 10 Times = 40 Cars ~ 26,794 rs

Total Amount – 29,494

Grid power spending amount – 2508

Revenue generation per day = 29,494 – 2,508

= 26,986 / day

Conclusion –By Integration of 3 Generation sources and Automation with the loads of Battery Swapping Station and DC Fast charging stations. We got good power balance stability.

Energy and Cost Efficiency is 88.5%

Project Applications

- Ev charging infrastructure

- Smart Grid integration

- Commercial buildings and Industrial applications [which have Solar, ESS and Grid sources]

- Remote Charging Stations