Synchronous Buck Converter

Parameters –

Battery voltage – 24v

Ripple inductor current – 10%

Ripple Capacitance voltage – 2%

Switching frequency – 250 khz

Duty cycle – 40%

--- power – 10w

Calculated –

Vout – 9.6v

Iout – 1.042A

Load resistance – 9.21ohms

Inductance Ripple current – 0.1042A

Capacitance ripple voltage – 0.192v

Inductance – 2.21uH

Capacitance – 271.35uF

T = 4us

Ton – 1.6us (40% duty cycle)

Output voltage

Output current

Inductor output current, Voltage pulse -1

Inductor output current, Voltage pulse -2

Capacitance output voltage, Voltage P	luse.